Noncovered Source Permit Review Summary (Modification)

Application No.:			0746-02 (application for modification)

Permit No.:				0746-01-N

Applicant:				UOP, LLC.

Facility Title:			Integrated Biorefinery (IBR) Pilot Facility

91-383 Kauhi Street

Kapolei, Hawaii  96707

UTM: 594,100 meters East, 2,356,520 meters North

Zone 4, NAD-83

Mailing Address:		UOP, LLC.

1250 W. Sam Houston Parkway S.

Houston, Texas  77042

Responsible Official:	Veronica M. May

Vice President and General Manager,

Renewable Energy and Chemicals

(847) 375-7187

Application Date:		Received on March 13, 2015

Proposed Project:

SICC - unknown

UOP proposes in this modification application to eliminate the construction and operation of the Upgrader Unit No. 2 and remove the unit from the permit.  The Upgrader Unit No. 2 was to be constructed after the successful operation of the RTP Unit for the purpose of converting the pyrolysis oil to usable transportation fuels.  Instead, after filtration and stabilization in the ion exchange unit (Upgrader Unit No. 1), the pyrolysis oil will be pumped into totes and shipped offsite for later processing at a separate facility.

Background

The initial noncovered permit application consisted of plans to construct and operate a biomass-to-fuel conversion pilot facility in Kapolei, Hawaii.  The facility is an integrated system utilizing pyrolysis and hydro-processing to convert various biomass feedstocks to transportation grade liquid fuels.

The UOP Integrated Biorefinery (IBR) Pilot project site is located in Kapolei, on the island of Oahu.  The site lies within the Campbell Industrial Park at 91-383 Kauhi Street, Kapolei, Hawaii.  The site is adjacent to the Hawaii Independent Energy refinery on a 4.5 acre parcel that is owned by the refinery.  The UOP facility, with a total footprint of approximately 1.24 acres, will occupy only the southwest corner of the parcel.  The project site is bounded by Komohana Street on the west and the Hawaii Independent Energy refinery on the south.  Beyond the vacant land to the north is Kauhi Street, and to the east is an unnamed road.

Description of Processes and Products

UOP’s IBR facility is a pilot-scale operation of a fully integrated process utilizing rapid thermal pyrolysis of various biomass feedstocks to produce stabilized pyrolysis oil for future (offsite) conversion to transportation-grade liquid fuels.  Biomass feedstock is shredded and dried offsite and will be shipped to the facility in 2,000 lb sacks, where they will be stored in the warehouse to await use.  The production capacity is 4 x 42 gallon of pyrolysis oil per day.

Biomass Processing Unit (RTP Unit)

The RTP unit converts dried biomass into liquid pyrolysis oil.  RTP (Rapid Thermal Processing) is a patented form of rapid pyrolysis, which is the thermal decomposition of biomass in the absence of oxygen.  The process occurs at moderate temperature with a high heat transfer rate to the biomass particles and a short, hot vapor residence time in the reaction zone.  The resulting product is a liquid bio-oil that has organic acids, contains oxygenated compounds, and has the potential to degrade over time.  The bio-oil (or pyrolysis oil) will be stored onsite to await upgrading to usable transportation fuels.

The RTP unit consists of the biomass feed hoppers, feed auger, pyrolysis reactor and cyclone, reheater with thermal media (sand), reheater cyclone, air blower (to provide char combustion air to the reheater), primary and secondary condensers, fiberbed filter with demister, knock-out pot, and baghouse.  Waste gases consist of reheater flue gas and, in an alternate operating scenario, non-condensable by-product gas.  Emissions from these sources consist of mainly nitrogen, carbon dioxide, oxygen, and water vapor, with small amounts of carbon monoxide and non-recoverable fuel gas.  The flue gas is passed through the baghouse and is vented from the baghouse stack at a rate of approximately 62.3 lbs/hr.  During normal operations, by-product gas from the condensers is passed through the fiberbed filter and recycled back to the RTP reactor at a rate of approximately 17.5 lbs/hr.  Alternatively, after passing through the fiberbed filter, flow of by-product gas may be split and sent to the reheater to be combusted or directly to the RTP exhaust stack to be emitted with the flue gas.  Since emissions are projected to be highest when the by-product gas is sent directly to the exhaust stack, this scenario is conservatively used for this emission estimate.

After production, the raw pyrolysis oil is filtered and stabilized via ion exchange and dehydration in Upgrader Unit No. 1, with a maximum throughput of approximately four barrels (168 gallons) per day.

Biofuel Upgrading Unit (Upgrader Unit Nos. 1 and 2)

The Biofuel Upgrading Unit first stabilizes and then converts the bio-oil to usable transportation fuels using a two-stage process.  The first stage (Upgrader Unit No. 1) involves filtering and ion exchange to reduce solids and metals to produce a stable pyrolysis oil.  The second stage (Upgrader Unit No. 2, to be eliminated per this modification application) involves the catalytic removal of oxygen from the oil via hydroprocessing followed by fractionation using distillation technology to yield a fuel gas, gasoline, and a minor quantity of heavy aromatics.

The Upgrader Unit No. 2 consisted of a de-oxygenation and hydrotreating reactor, a hydrogen separation system, and a distillation system.  The waste gases from the fractionation process consisted mainly of hydrogen, carbon dioxide, and carbon monoxide, along with non-condensable fuel gases.  The gases were to be vented from the Upgrader stack to the atmosphere at a rate of approximately 7.0 lb per hour.

Operating Schedule

For the pilot study, UOP will operate the facility from the first quarter of 2015 until the second quarter of 2015.  The RTP unit will be operated for an average of approximately fifteen (15) days per month at a maximum of twenty-four (24) hours per day.  To be conservative, UOP requested to operate 365 days per year at twenty-four (24) hours per day, for a maximum of 8,760 hours per twelve month (12-month) period.

The application fee of $100.00 for a modification to a noncovered source was received and processed.

Equipment Description:

1.	Biomass Processing Unit (RTP Unit)

Make: 						Proprietary unit fabricated by Zeton, Inc.

Serial Number: 				will be submitted upon purchase

Input

Biomass feedstocks used: 	wood, sugar cane, bagasse, corn stover, guinea grass, 								switchgrass, and/or algae; pre-dried to six (6) percent 								moisture content

Biomass input:				1.0 bone-dry metric tons per day (BDMTPD)

Output

Flue gas flow:				62.3 lb/hr

Byproduct gas flow			17.5 lb/hr

Max. pyrolysis oil production:	1,584 lb/day

Max. operating hours:			8,760 hrs/year

2.	Biofuel Upgrading Unit (Upgrader Unit No. 1) – used to filtrate and stabilize the raw pyrolysis oil via ion exchange.

Air Pollution Controls:

1. 	Baghouse

The baghouse serving the RTP unit is designed to control particulate emissions with at least 99% efficiency.

Make:						Donaldson Torit

Model:						DLMV 4/7 type W (H+weather cowl) base model

Filter material:				P84 polyamide composite high temperature, high efficiency 								filter

Nominal airflow range:		215-555 cfm

Cloth area:					43 ft2

Particulate removal efficiency: 	≥ 99%

2.	Fiberbed Filter/Demister

Downstream of the condensers, a demister vessel and a system of fiber bed filters enable the coalescence and collection of the aerosols in the stream of non-condensable by-product vapor.

3.	Cyclones

A high efficiency cyclone is used to separate product and byproduct vapors from the solid char in the RTP reactor and a second cyclone is used to separate sand from flue gas in the RTP reheater to maximize solids retention in the RTP unit.  Solids are sequestered in enclosed bins.

Both units have been designed to recover and recycle the majority of process gases to maximize efficiency and minimize emissions.  The units are strictly monitored during operation for leaks using pressure and flow transducers as well as routine visual inspections.

Exemptions:

Storage Tanks – exempted per HAR §11-60.1-62(d)(3) for storage tanks of VOCs under

40,000 gallons.

Pyrolysis oil:				3 x 260 gallons

Pyroysis oil (filtered):		1 x 4,000 gallons

Pyrolysis oil (stabilized):	1 x 8,500 gallons

1 x 2,000 gallons

Ethanol (95%):			3 x 500 gallons

Brine (10%):				1 x 1,600 gallons

1 x 1,050 gallons

1 x 750 gallons

Sulfuric acid (5%);		1 x 1,060 gallons

1 x 1,050 gallons

2 x 500 gallons

1 x 2,500 gallons

Applicable Requirements:

Hawaii Administrative Rules (HAR)

Title 11, Chapter 59		Ambient Air Quality Standards

Title 11, Chapter 60.1		Air Pollution Control

Subchapter 1		General Requirements

Subchapter 2		General Prohibitions

HAR 11-60.1-32	Visible Emissions

Subchapter 4		Noncovered Sources

Subchapter 6		Fees for Covered Sources, Noncovered Sources, and Agricultural 							Burning

HAR 11-60.1-111	Definitions

HAR 11-60.1-117	General Fee Provisions for Noncovered Sources

HAR 11-60.1-118	Application Fees for Noncovered Sources

HAR 11-60.1-119	Annual Fees for Noncovered Sources

Non-Applicable Requirements:

Hawaii Administrative Rules (HAR)

Title 11, Chapter 60.1		Air Pollution Control

Subchapter 5		Covered Sources

Subchapter 7		Prevention of Significant Deterioration

Subchapter 8		Standards of Performance for Stationary Sources

Subchapter 9		Hazardous Air Pollutant Sources

Federal Requirements

40 CFR Part 52.21 - Prevention of Significant Deterioration of Air Quality

40 CFR Part 60 – Standards of Performance for New Stationary Sources (NSPS)

40 CFR Part 61 – National Emission Standards for Hazardous Air Pollutants (NESHAPS)

40 CFR Part 63 – National Emission Standards for Hazardous Air Pollutants for Source Categories (Maximum Achievable Control Technologies (MACT) Standards)

Prevention of Significant Deterioration (PSD):

This source is not a major stationary source nor are there modifications proposed that by itself constitute a major stationary source that is subject to PSD review.  Therefore, PSD is not applicable.

Best Available Control Technology (BACT):

A Best Available Control Technology (BACT) analysis is applicable only to new noncovered sources and modifications to noncovered sources that have the potential to emit or increase emissions above significant levels as defined in HAR §11-60.1-1.  A BACT analysis is not applicable since the potential to emit is less than the significant levels as shown in the table below.

BACT Analysis

| Pollutant | Potential Emissions (tpy) | Significant Levels (tpy) | BACT Applicable? |
| --- | --- | --- | --- |
| NOx | 0.027 | 40 | No |
| SOx | 0 | 40 | No |
| CO | 22.08 | 100 | No |
| PM | 0.0005 | 25 | No |
| VOC | 4.02 | 40 | No |

Air Emissions Reporting Requirements (AERR):

40 CFR Part 51, Subpart A – Air Emissions Reporting Requirements, is based on the emissions of criteria air pollutants from Type B point sources (as defined in 40 CFR Part 51, Subpart A), that emit at the AERR triggering levels as shown in the table below.

AERR/In-house Reporting Applicability

| Pollutant | Type B AERR Triggering Levels 1 (tpy) | Pollutant | In-house Total Facility Triggering Levels 1 (tpy) | Potential Emissions (tpy) |
| --- | --- | --- | --- | --- |
| NOx | ≥100 | NOx | ≥25 | 0.027 |
| SOx | ≥100 | SOx | ≥25 | 0 |
| CO | ≥1000 | CO | ≥250 | 22.08 |
| PM10/ PM2.5 | ≥100/100 | PM/PM10 | ≥25/25 | PM, PM10, PM2.5 = 0.0005 |
| VOC | ≥100 | VOC | ≥25 | 4.02 |
|  |  | HAPs | ≥5 | 0.27 |

1	Based on potential emissions

This facility does not emit at the AERR triggering levels.  Therefore, AERR is not applicable.

Although AERR for the facility is not triggered, the Clean Air Branch requests annual emissions reporting from those facilities that have facility-wide emissions of a single air pollutant exceeding in-house triggering levels.  Annual emissions reporting for the facility will not be required for

in-house recordkeeping purposes since there are no pollutants exceeding the in-house triggering levels.

Compliance Assurance Monitoring (CAM):

40 CFR Part 64

The purpose of Compliance Assurance Monitoring (CAM) is to provide a reasonable assurance that compliance is being achieved with large emission units that rely on air pollution control device equipment to meet an emission limit or standard.  Pursuant to 40 CFR Part 64, for CAM to be applicable, the emission units must: (1) be located at a major source; (2) be subject to an emissions limit or standard; (3) use a control device to achieve compliance; (4) have precontrol potential emissions that are 100% of the major source level; and (5) not otherwise be exempt from CAM.  CAM is or is not applicable respectively to the emission units because all of (1) through (5) above apply or any (1) through (5) above do not apply.  This facility is not subject to CAM because it is a non-major source.

Synthetic Minor Source:

This source is not a synthetic minor since this source is still a non-major source if the facility is operated continuously at 8,760 hr/yr.

Alternate Operating Scenarios:

As discussed in the description of the Biomass Processing Unit (RTP Unit), after passing through the fiberbed filter, instead of being recycled back to the reactor, flow of by-product gas may be split and sent to the reheater to be combusted or directly to the RTP exhaust stack to be emitted with the flue gas.

Project Emissions:

Potential to emit (PTE) calculations for the proposed operations were performed using design data provided by UOP.  This data for the composition of exhaust flue gases and byproduct gases (in percent by weight) were estimated by UOP using small scale laboratory testing, mass balance equations, and advance chemical engineering models.  UOP also provided flow data based on the maximum production rate of four (4) barrels per day of pyrolysis oil produced.  This data was then converted to lbs/day and tons/year based on the proposed operating schedule.  Though different feedstocks will yield different emissions during pyrolysis, the test data used in this analysis was for guinea grass.  This is considered to be conservative because it will produce the highest emissions of carbon monoxide, which is calculated to be the highest emitted criteria pollutant by a wide margin.  PTE calculations were based on the maximum production, and assume the optional scenario of diverting the by-product gas stream straight to the exhaust stack instead of recycling it to the reactor or combusting it in the reheater.  Potential emissions were based on 8,760 hours per year operation.

Potential Emissions for the UOP Integrated Biorefinery (IBR) Pilot Facility

| Source | NOx (tpy) | SOX (tpy) | CO (tpy) | PM (tpy) | VOC (tpy) | HAPs (tpy) | CO2 (tpy) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RTP unit (UOP-1) | 0.027 | 0 | 22.08 | 0.0005 | 4.02 | 0.27 | 52.63 |

Ambient Air Quality Assessment:

An additional ambient air quality impact analysis (AAQIA) was not performed for this modification application since the initial permit application’s AAQIA included emissions from the RTP Unit only.  The Upgrader Unit No. 2 was not included in the initial AAQIA due to its very low emissions (less than 1 tpy and 0.1 lb/hr for the criteria pollutants).  Therefore, this modification application to delete the Upgrader Unit No. 2 does not change the results of the initial AAQIA, shown in the table below.

Predicted Ambient Air Quality Impacts

| Pollutant | Averaging Period | Maximum Modeled Concentration (µg/m3) | Background Concentration1 (µg/m3) | Background Concentration1 (ppm) | Total Impact (µg/m3) | SAAQS (µg/m3) | Percent of SAAQS2 (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NO2 | 1-hr | 5.2376 | 58.28 | 0.031 | 63.52 | 188 (100 ppb) | 33.8 |
|  | Annual | 37.5 | 7.52 | 0.004 | 45.02 | 70 | 64.3 |
| CO | 1-hr | 4158 | 4236.5 | 3.7 | 8394.5 | 10000 | 83.9 |
|  | 8-hr | 2910.6 | 1374 | 1.2 | 4284.6 | 5000 | 85.7 |
| PM10 | 24-hr | 0.0377 | 37 |  | 37.04 | 150 | 24.7 |
|  | Annual | 0.0189 | 16 |  | 16.02 | 50 | 32.0 |
| PM2.5 | 24-hr | 0.0377 | 25 |  | 25.04 | 35 | 71.5 |
|  | Annual | 0.0189 | 5.5 |  | 5.52 | 15 | 36.8 |

1	Background concentration data based on the following monitoring stations:

Kapolei (2009) – NOx, CO, PM10, PM 2.5,

2	Only the State Ambient Air Quality Standards (SAAQS) are shown as they are the same or more restrictive than the National Ambient Air Quality Standards (NAAQS)

3	Calculated using the Ozone Limiting Method (OLM)

NO2 = 0.1(NOx)max + (NO2/O3) x (O3)ambient = 0.1 x 1.05 µg/m3 + 46/48 x 39 µg/m3 = 37.5 µg/m3

Sand Island -2009 – (O3)ambient = 39 µg/m3

Significant Permit Conditions:

Significant permit conditions consisted of the following for the UOP Integrated Biorefinery (IBR) Pilot Facility:

Attachment II, Special Condition No. A.1

1.	This permit encompasses the following equipment and associated appurtenances for the Integrated Biorefinery (IBR) Pilot Facility:

a.	Biomass Processing Unit (RTP Unit) – Zeton, Inc.:

i.	Feed bin;

ii.	RTP reactor;

iii.	Reactor cyclone;

iv.	Reheater (electric);

v.	Reheater cyclone;

vi.	Baghouse – Donaldson Torit, model DLMV 4/7 type W (H + weather cowl);

vii.	Condensers; and

viii.	Demister and Fiberbed Filter.

b.	Biofuel Ungrading Unit (Upgrader Unit No.1) – used to filtrate and stabilize the raw pyrolysis oil using ion exchange.

Attachment II, Special Condition No. B.1

1.	Biomass Feedstock

Only wood, sugar cane, bagasse, corn stover, guinea grass, switch grass, and/or algae; pre-dried to a six (6) percent moisture content, shall be used as biomass feedstock to be converted into pyrolysis oil.

Attachment II, Special Condition No. B.2

- 2.	Emissions Limitations

- For any six (6) minute averaging period, the RTP Unit shall not exhibit visible emissions of twenty (20) percent opacity or greater, except as follows:  during start-up, shutdown, or equipment breakdown, the RTP Unit may exhibit visible emissions not greater than

- sixty (60) percent opacity for a period aggregating not more than six (6) minutes in any

- sixty (60) minute period.

Conclusion and Recommendation:

Recommend issuing a modification to the UOP Integrated Biorefinery (IBR) Pilot Facility, incorporating the significant changes shown above into the permit, subject to all applicable state and federal air pollution requirements.  This permit shall supersede NSP No. 0746-01-N issued on August 30, 2011 in its entirety.

Reviewer: Darin Lum

Date:	4/2015