Noncovered Source Permit Review Summary (Initial)

Application File No.:		0746-01

Permit No.:				0746-01-N

Applicant:				UOP, LLC.

Facility Title:			Integrated Biorefinery (IBR) Pilot Facility

91-383 Kauhi Street

Kapolei, Hawaii  96707

UTM: 594,100 meters East, 2,356,520 meters North

Mailing Address:		UOP, LLC.

25 E. Algonquin Rd.

Des Plaines, IL  60017

Responsible Official:	James E. Rekoske

Vice President and General Manager,

Renewable Energy and Chemicals

(847) 391-2758

Application Date:		Received on June 13, 2011,

additional information dated August 1, 2011

Proposed Project:

SICC  - unknown

This is an application to construct and operate a biomass-to-fuel conversion pilot facility in Kapolei, Hawaii.  The facility is an integrated system utilizing pyrolysis and hydro-processing to convert various biomass feedstocks to transportation grade liquid fuels.

The UOP Integrated Biorefinery (IBR) Pilot project site is located in Kapolei, on the island of Oahu.  The site lies within the Campbell Industrial Park at 91-383 Kauhi Street, Kapolei, Hawaii.  The site is adjacent to the Tesoro Refinery on a 4.5 acre parcel that is owned by the refinery but is currently vacant.  The UOP facility, with a total footprint of approximately 1.24 acres, will occupy only the southwest corner of the parcel.  The project site is bounded by Komohana Street on the west and the Tesoro refinery on the south.  Beyond the vacant land to the north is Kauhi Street, and to the east is an unnamed road.

Description of Processes and Products

UOP’s IBR facility is a pilot-scale operation of a fully integrated process to convert high impact biomass to fuels including gasoline, diesel, and jet-range hydrocarbons.  The feeds will be converted to fuels via integrated pyrolysis and hydro-conversion, which will be accomplished with two distinct units: a pyrolysis unit and an upgrader unit.  Biomass feedstock is shredded and dried offsite and will be shipped to the facility in 2,000 lb sacks, where they will be stored in the warehouse to await use.  The production capacity is 4 x 55 drums of fuel per day.

Biomass Processing Unit (RTP Unit)

The RTP unit converts dried biomass into liquid pyrolysis oil.  RTP (Rapid Thermal Processing) is a patented form of rapid pyrolysis, which is the thermal decomposition of biomass in the absence of oxygen.  The process occurs at moderate temperature with a high heat transfer rate to the biomass particles and a short, hot vapor residence time in the reaction zone.  The resulting product is a liquid bio-oil that has organic acids, contains oxygenated compounds, and has the potential to degrade over time.  The bio-oil (or pyrolysis oil) will be stored onsite to await upgrading to usable transportation fuels.

The RTP unit consists of the biomass feed hoppers, feed auger, pyrolysis reactor and cyclone, reheater with thermal media (sand), reheater cyclone, air blower (to provide char combustion air to the reheater), primary and secondary condensers, fiberbed filter with demister, and baghouse.  Waste gases consist of reheater flue gas and non-condensable by-product gas.  Emissions from these sources consist of mainly nitrogen, carbon dioxide, oxygen, and water vapor, with small amounts of carbon monoxide and non-recoverable fuel gas.  The flue gas is passed through the baghouse and is vented from the baghouse stack at a rate of approximately 62.3 lbs/hr.  By-product gas is emitted after the fiberbed filter at a rate of approximately

17.5 lbs/hr.

Biofuel Upgrading Unit (Upgrader Unit)

The Upgrader unit first stabilizes and then converts the bio-oil to usable transportation fuels using a two-stage process.  The first stage involves filtering and ion exchange to reduce solids and metals to produce a fuel suitable for burning in a furnace for heating purposes or in a stationary diesel engine to produce electricity.  The second stage involves the catalytic removal of oxygen from the oil via hydroprocessing followed by fractionation using distillation technology to yield a fuel gas, gasoline, and a minor quantity of heavy aromatics.

The Upgrader unit will consists of a de-oxygenation and hydrotreating reactor, a hydrogen separation system, and a distillation system.  The waste gases from the fractionation process consists mainly of hydrogen, carbon dioxide, and carbon monoxide, along with

non-condensable fuel gases.  The gases will be vented from the Upgrader stack to the atmosphere at a rate of approximately 7.0 lb per hour.

Operating Schedule

For the pilot study, UOP will operate the facility from the first quarter of 2012 until early to mid 2014.  While the RTP unit will operate throughout this timeframe, the Upgrader unit will not come online until early 2013.  The RTP unit will be operated for an average of approximately

fifteen (15) days per month at a maximum of twenty-four (24) hours per day.  The Upgrader unit will be operated for an average of approximately twenty-five (25) days per month at a maximum of 24 hours per day.  To be conservative, UOP requests to be permitted to operate 365 days per year at twenty-four (24) hours per day, for a maximum of 8,760 hours per twelve month

(12-month) period, for both the RTP unit and the Upgrader.  The emissions calculations for this report conservatively assume this will be the case and that both units will be operating simultaneously, though this will only occur from 2013 to 2014.

The application fee of $150.00 for an initial noncovered source permit was received and processed.

Equipment Description:

1.	Biomass Processing Unit (RTP Unit)

Make: 						Proprietary unit fabricated by Zeton, Inc.

Serial Number: 				will be submitted upon purchase

Input

Biomass feedstocks used: 	wood, sugar cane, bagasse, corn stover, guinea grass, 								switchgrass, and/or algae; pre-dried to six (6) percent 								moisture content

Biomass input:				1.0 bone-dry metric tons per day (BDMTPD)

Output

Flue gas flow:				62.3 lb/hr

Byproduct gas flow			17.5 lb/hr

Max. pyrolysis oil production:	1,584 lb/day

Max. operating hours:			8,760 hrs/year

2.	Biofuel Upgrading Unit (Upgrader Unit)

Make:						Fabricated by Zeton, Inc.

Input

Pyrolysis oil:					1,584 lb/day

Hydrogen:					96 lb/day

Output

Byproduct gas flow (vented)	7.0 lb/hr

Gasoline production:			408 lb/day

Diesel production:			192 lb/day

Max. operating hours:			8,760 hrs/year

Air Pollution Controls:

1. 	Baghouse

The baghouse serving the RTP unit is designed to control particulate emissions with at least 99% efficiency.

Make:						Donaldson Torit

Model:						DLMV 4/7 type W (H+weather cowl) base model

Filter material:				P84 polyamide composite high temperature, high efficiency 								filter

Nominal airflow range:		215-555 cfm

Cloth area:					43 ft2

Particulate removal efficiency: 	≥ 99%

2.	Fiberbed Filter/Demister

Downstream of the condensers, a demister vessel and a system of fiber bed filters enable the coalescence and collection of the aerosols in the stream of non-condensable by-product vapor.

3.	Cyclones

A high efficiency cyclone is used to separate product and byproduct vapors from the solid char in the RTP reactor and a second cyclone is used to separate sand from flue gas in the RTP reheater to maximize solids retention in the RTP unit.  Solids are sequestered in enclosed bins.

Both units have been designed to recover and recycle the majority of process gases to maximize efficiency and minimize emissions.  The units are strictly monitored during operation for leaks using pressure and flow transducers as well as routine visual inspections.

Exemptions:

Storage Tanks – exempted per HAR §11-60.1-62(d)(2) for storage tanks of VOCs under

40,000 gallons.

Pyrolysis oil:				3 x 260 gallons

Pyroysis oil (filtered):		1 x 2,500 gallons

Pyrolysis oil (stabilized):	1 x 8,500 gallons

Ethanol (95%):			1 x 300 gallons

1 x 500 gallons

1 x 1,000 gallons

Brine (10%):				1 x 1,600 gallons

1 x 1,050 gallons

1 x 750 gallons

Sulfuric acid (5%);		1 x 1,060 gallons

1 x 1,050 gallons

2 x 500 gallons

1 x 750 gallons

Diesel fuel 				1 x 8,500 gallons

2 x 55 gallons

Gasoline:				1 x 8,500 gallons

Nitrogen (liquid):			440 lbs

Applicable Requirements:

Hawaii Administrative Rules (HAR)

Title 11, Chapter 59		Ambient Air Quality Standards

Title 11, Chapter 60.1		Air Pollution Control

Subchapter 1		General Requirements

Subchapter 2		General Prohibitions

HAR 11-60.1-32	Visible Emissions

Subchapter 4		Noncovered Sources

Subchapter 6		Fees for Covered Sources, Noncovered Sources, and Agricultural 							Burning

HAR 11-60.1-111	Definitions

HAR 11-60.1-117	General Fee Provisions for Noncovered Sources

HAR 11-60.1-118	Application Fees for Noncovered Sources

HAR 11-60.1-119	Annual Fees for Noncovered Sources

Non-Applicable Requirements:

Hawaii Administrative Rules (HAR)

Title 11, Chapter 60.1		Air Pollution Control

Subchapter 5		Covered Sources

Subchapter 7		Prevention of Significant Deterioration

Subchapter 8		Standards of Performance for Stationary Sources

Subchapter 9		Hazardous Air Pollutant Sources

Federal Requirements

40 CFR Part 52.21 - Prevention of Significant Deterioration of Air Quality

40 CFR Part 60 – Standards of Performance for New Stationary Sources (NSPS)

40 CFR Part 61 – National Emission Standards for Hazardous Air Pollutants (NESHAPS)

40 CFR Part 63 – National Emission Standards for Hazardous Air Pollutants for Source Categories (Maximum Achievable Control Technologies (MACT) Standards)

Prevention of Significant Deterioration (PSD):

This source is not a major stationary source nor are there modifications proposed that by itself constitute a major stationary source that is subject to PSD review.  Therefore, PSD is not applicable.

Best Available Control Technology (BACT):

A Best Available Control Technology (BACT) analysis is applicable only to new noncovered sources and modifications to noncovered sources that have the potential to emit or a net emissions increase above significant levels as defined in HAR §11-60.1-1.  A BACT analysis is not applicable since the potential to emit is less than the significant levels as shown in Table 1 below.

Table 1 – BACT Analysis

| Pollutant | Potential Emissions (tpy) | Significant Levels (tpy) | BACT Applicable? |
| --- | --- | --- | --- |
| NOx | 0.065 | 40 | No |
| SOx | 0.037 | 40 | No |
| CO | 22.22 | 100 | No |
| PM/PM10 | 0.0130/0.0130 | 25/15 | No |
| VOC | 0.037 | 40 | No |

Consolidated Emissions Reporting Rule (CERR):

40 CFR Part 51, Subpart A - Emission Inventory Reporting Requirements, determines CER based on the emissions of criteria air pollutants from Type B point sources (as defined in

40 CFR Part 51, Subpart A), that emit at the CER triggering levels as shown in Table 2 below.

Table 2 - CERR/In-house Reporting Applicability

| Pollutant | Type B CER Triggering Levels 1 (tpy) | Pollutant | In-house Total Facility Triggering Levels 2 (tpy) | Potential Emissions (tpy) |
| --- | --- | --- | --- | --- |
| NOx | ≥100 | NOx | ≥25 | 0.065 |
| SOx | ≥100 | SOx | ≥25 | 0.037 |
| CO | ≥1000 | CO | ≥250 | 22.22 |
| PM10/ PM2.5 | ≥100/100 | PM/PM10 | ≥25/25 | PM, PM10, PM2.5 = 0.0130 |
| VOC | ≥100 | VOC | ≥25 | 4.67 |
|  |  | HAPs | ≥5 | 0.88 |

1	Based on actual emissions

2	Based on potential emissions

This facility does not emit at the CER triggering levels.  Therefore, CER requirements are not applicable.

Although CER for the facility is not triggered, the Clean Air Branch requests annual emissions reporting from those facilities that have facility-wide emissions of a single air pollutant exceeding in-house triggering levels.  Annual emissions reporting for the facility will not be required for

in-house recordkeeping purposes since there are no pollutants exceeding the in-house triggering levels.

Compliance Assurance Monitoring (CAM):

40 CFR Part 64

The purpose of Compliance Assurance Monitoring (CAM) is to provide a reasonable assurance that compliance is being achieved with large emission units that rely on air pollution control device equipment to meet an emission limit or standard.  Pursuant to 40 CFR Part 64, for CAM to be applicable, the emission units must: (1) be located at a major source; (2) be subject to an emissions limit or standard; (3) use a control device to achieve compliance; (4) have precontrol potential emissions that are 100% of the major source level; and (5) not otherwise be exempt from CAM.  CAM is or is not applicable respectively to the emission units because all of (1) through (5) above apply or any (1) through (5) above do not apply.  This facility is not subject to CAM because it is a non-major source.

Synthetic Minor Source:

This source is not a synthetic minor since this source is still a non-major source if the facility is operated continuously at 8,760 hr/yr.

Alternate Operating Scenarios:

None proposed.

Project Emissions:

Potential to emit (PTE) calculations for the proposed operations were performed using design data provided by UOP.  These data for the composition of exhaust flue gases and byproduct gases (in percent by weight) were estimated by UOP using small scale laboratory testing, mass balance equations, and advance chemical engineering models.  UOP also provided flow data based on the maximum production rate of four barrels per day of pyrolysis oil produced.  This data was then converted to lbs/day and tons/year based on the proposed operating schedule.  Though different feedstocks will yield different emissions during pyrolysis, the test data used in this analysis was for guinea grass.  This is considered to be conservative because it will produce the highest emissions of carbon monoxide, which is calculated to be the highest emitted criteria pollutant by a wide margin.  PTE calculations were based on the maximum production at each of the two emission points.

Table 3 – Potential Emissions for the UOP Integrated Biorefinery (IBR) Pilot Facility

| Source | NOx (tpy) | SOX (tpy) | CO (tpy) | PM (tpy) | VOC (tpy) | HAPs (tpy) | CO2 (tpy) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RTP unit (UOP-1) | 0.027 | 0 | 22.08 | 0.0005 | 4.02 | 0.27 | 52.63 |
| Upgrader unit (UOP-2) | 0.038 | 0.037 | 0.14 | 0.0125 | 0.65 | 0.61 | 27.36 |
| Total | 0.065 | 0.037 | 22.22 | 0.0130 | 4.67 | 0.88 | 79.99 |

Ambient Air Quality Assessment:

The applicant conducted an ambient air quality impact analysis (AAQIA) to determine the emissions impact on the ambient air quality from the RTP Unit.  An analysis for the Upgrader Unit was not conducted due to its very low emissions (less than 1 tpy and 0.1 lb/hr for the criteria pollutants).  The analysis used the EPA SCREEN3 model to quantify ambient air pollutant impacts in the surrounding area.

The assumptions used in the SCREEN3 model included the following:

●	Simple terrain option

●	Urban dispersion parameters

●	Full meteorology

●	EPA scaling factor of 0.9, 0.7, and 0.4 for the 3-hr, 8-hr and 24-hr concentrations, respectively

●	State of Hawaii default scaling factor of 0.2 for the annual concentration

Background concentration data were based on the Kapolei Station (2009).

Downwash effects from the RTP Unit’s structure were included in the modeling analysis.

Discrete SCREEN3 receptors were set every 100 meters from the stack.

Table 4 shows the potential downwash-inducing structures.  Table 5 presents the potential emission rates and stack parameters used in the ambient air quality impact analysis.  Table 6 and Table 7 are the modeled results and maximum modeled impacts for the RTP Unit, respectively.

The predicted ambient air quality impacts are shown in Table 8.  The table demonstrates that the impacts of NO2, CO and PM10 from the RTP Unit plus background air quality levels should not cause or contribute to a violation of any State or National ambient air quality standard.

Table 4 - Potential Downwash-Inducing Structures

| Structure | Height (Hb) (m) | Length (Lb) (m) | Width (Wb) (m) | Projected Width (PW) (m) | L (m) | Hg =Hb + 1.5L (m) | 5L | Is structure within 5L distance to stack? |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RTP | 10.9728 | 7.3152 | 3.6576 | 8.1786 | 8.1786 | 23.2407 | 40.893 | Yes |

Notes:

PW = (Lb2 + Wb2)0.5

L is the lessor of Hb or PW

Table 5 - Emission Rates and Stack Parameters

| Equipment |  | Emission Rates |  |  |  | Stack Parameters |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NOx (g/s) | CO (g/s) | PM10 (g/s) | Height (m) | Temp. (K) | Velocity (m/s) | Flow Rate (m3/s) | Diameter (m) |
| RTP Unit | 0.0008 | 0.6351 | 1.44E-05 | 14.63 | 422 | 2.47 | 0.0109 | 0.0758 |

Table 6 - Modeled Results

| Averaging Period | Conversion Factor | Normalized Concentration for 1 g/s (µg/m3 per g/s) |
| --- | --- | --- |
| 1-hr | na | 6547 |
| 3-hr | 0.9 | 5892.3 |
| 8-hr | 0.7 | 4582.9 |
| 24-hr | 0.4 | 2618.8 |
| Annual | 0.2 | 1309.4 |

Bolded entry is modeled result

Table 7 - Maximum Modeled Impacts

| Pollutant | Averaging Period | Emission Rate (g/s) | Maximum Modeled Concentration (µg/m3) |
| --- | --- | --- | --- |
| NO2 | 1-hr | 0.0008 | 5.2376 |
|  | Annual | 0.0008 | 1.0475 |
| CO | 1-hr | 0.6351 | 4158 |
|  | 8-hr | 0.6351 | 2910.6 |
| PM10 | 24-hr | 1.44E-05 | 0.0377 |
|  | Annual | 1.44E-05 | 0.0189 |
| PM2.5 | 24-hr | 1.44E-05 | 0.0377 |
|  | Annual | 1.44E-05 | 0.0189 |

Table 8 - Predicted Ambient Air Quality Impacts

| Pollutant | Averaging Period | Maximum Modeled Concentration (µg/m3) | Background Concentration1 (µg/m3) | Background Concentration1 (ppm) | Total Impact (µg/m3) | SAAQS (µg/m3) | Percent of SAAQS2 (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NO2 | 1-hr | 5.2376 | 58.28 | 0.031 | 63.52 | 188 (100 ppb) | 33.8 |
|  | Annual | 37.5 | 7.52 | 0.004 | 45.02 | 70 | 64.3 |
| CO | 1-hr | 4158 | 4236.5 | 3.7 | 8394.5 | 10000 | 83.9 |
|  | 8-hr | 2910.6 | 1374 | 1.2 | 4284.6 | 5000 | 85.7 |
| PM10 | 24-hr | 0.0377 | 37 |  | 37.04 | 150 | 24.7 |
|  | Annual | 0.0189 | 16 |  | 16.02 | 50 | 32.0 |
| PM2.5 | 24-hr | 0.0377 | 25 |  | 25.04 | 35 | 71.5 |
|  | Annual | 0.0189 | 5.5 |  | 5.52 | 15 | 36.8 |

1	Background concentration data based on the following monitoring stations:

Kapolei (2009) – NOx, CO, PM10, PM 2.5,

2	Only the State Ambient Air Quality Standards (SAAQS) are shown as they are the same or more restrictive than the National Ambient Air Quality Standards (NAAQS)

3	Calculated using the Ozone Limiting Method (OLM)

NO2 = 0.1(NOx)max + (NO2/O3) x (O3)ambient = 0.1 x 1.05 µg/m3 + 46/48 x 39 µg/m3 = 37.5 µg/m3

Sand Island -2009 – (O3)ambient = 39 µg/m3

Significant Permit Conditions:

Significant permit conditions consisted of the following for the UOP Integrated Biorefinery (IBR) Pilot Facility:

1. 	Biomass Feedstock

Only wood, sugar cane, bagasse, corn stover, guinea grass, switch grass, and/or algae; pre-dried to a 6 percent moisture content, shall be used as biomass feedstock to be converted into transportation grade liquid fuels.

2.	Opacity limit of 20% maximum for the RTP Unit and Upgrader Unit, except during start-up, shutdown, or equipment breakdown.

Conclusion and Recommendation:

Recommend issuance of Noncovered Source Permit (NSP) No. 0746-01-N for the UOP Integrated Biorefinery (IBR) Pilot Facility, subject to the incorporation of the significant permit conditions noted above.  The facility complies with all Federal and State Ambient Air Quality Standards.

Reviewer: Darin Lum

Date:	8/2011